#### Description

DH984A is a 40” PWM or DAC voice output with high quality speech ,2 input pin and 6 I/O pin simple speech.

#### Functions

- MAX voice file : 700

- MAX Groups : 64

- MAX Step : 700

- Signal step mute length : 0.64 s ( 6k sample rate )

- Operating Voltage range: 2.4V ~ 5V

- Total Voice Duration : 40” (240K Samples)

- Input pin : 2

- I/O pin : 6

- Voice output : PWM and DAC

- Sequential Key : TG1 -> 32 Groups.

TG2 -> 21 Groups.

TG5~TG12-> 1 Group

- Debounce time : 50 us or 10 ms

- On/Off function : only for TG1 & TG2

- Trigger mode (for all Input pins) :

- Edge/Level

- Hold/Unhold

- Retrigger/Irretrigger

- Output status (for each Output pin):

- Stand by Status.

- Busy Low Active.

- Busy High Active.

- LED Flash at 6 Hz.

- LED Flash at 3 Hz.

- Play rate level :

DH984A can provide different play rate in one code as follow

| Play rate(K) | Play rate(K) |
| --- | --- |
| 3 | 6 |
| 3.2 | 6.4 |
| 3.31 | 6.85 |
| 3.42 | 7.38 |
| 3.55 | 8 |
| 3.69 | 8.72 |
| 3.84 | 9.6 |
| 4 | 10.66 |
| 4.17 | 12 |
| 4.36 | 13.71 |
| 4.57 | 16 |
| 4.8 | 19.2 |
| 5.05 | 24 |
| 5.33 |  |
| 5.64 |  |

#### DH984A Pad Location

CHIP SIZE: (0, 0) (1730,1196) UM

- The IC substrate should be connect to vss

DH984A

#### DH984A DC character

| ##### Item | # Symbol | # Min | ##### Typ | # Max | Unit | Condition |
| --- | --- | --- | --- | --- | --- | --- |
| Operating voltage | VDD | 2.4 | 3.0 | 5 | V |  |
| Standby current | Isb |  | 1 | 5 | uA | VDD=3V,no load |
| Operating current | Iop |  | 600 |  | uA | VDD=3V,no load |
|  |  |  | 1 |  | mA | VDD=4.5V,no load |
| Drive current of O | Iod |  | 5 |  | mA | VDD=3V |
| Sink current of O | Ios |  | 10 |  | mA | VDD=3V |
| Drive current of PWM | Iod |  | 200 |  | mA | VDD=3V,VOUT=1.5V |
| Sink current of PWM | Ios |  | 200 |  | mA | VDD=3V,VOUT=1.5V |

#### DH984A Application Circuit

| DH984A | - System Power 2.4~3.6V - PWM1 & PWM2 directly drive SpeTFer |
| --- | --- |
| DH984A | - System Power 2.4~5V - PWM1 & PWM2 directly drive SpeTFer |
| DH984A | - System Power 2.4~3.6V - DAC Output , Connected with 8050 & Resistor driving SpeTFer |

#### DH984A Package Information

| DH984A-S08A DH984A-SOP8 | PIN; NAME. 1; TG2. 2; TG1. 3; TG3. 4; VDDL. 5; VSS. 6; PWM1. 7; VDD. 8; PWM2/DAC. |
| --- | --- |
| DH984A-D14 DH984A-I14 | PIN; NAME. 1; PWM2/DAC. 2; PWM1. 3; VDD5. 4; VDD. 5; VSS. 6; TG12. 7; TG11. 8; TG8. 9; NC. 10; TG2. 11; TG1. 12; TG5. 13; TG6. 14; TG7. |

- Writer Mapping Description

Writer Pin : TG2,TG1,TG5,VSS,PWM2,PWM1,VDD5,VDD

Write Board Revised from 2M Writer Board --- Pin Assignment Reference

| OTP Writer Power Board Pin Mapping Table |  |  |  |
| --- | --- | --- | --- |
| DIP 48 | Pin No-Pin Name ( 2M 40Pin COB) | Pin No-Pin Name (1M 40 Pin COB) | Pin No-PinName (512K 40 Pin COB) |
| 10 | 5 – RST_N |  |  |
| 11 | 12- GND | 12-GND | 12-GND |
| 12 | 9-TG4 | 16-PWM2 | 16-PWM2 |
| 13 | 8-TG3 | 18-PWM1 | 18-PWM1 |
| 14 | 7-VDD | 19,20-VDD5,VDD | 17-VDD |
| 36 | 13-TEST |  |  |
| 37 | 6-VPP | 1-TG2 | 1-TG2 |
| 38 | 10-TG11 | 2-TG1 | 10-TG11 |
| 39 | 11-TG12 | 3-TG5 | 11-TG12 |

- Writer Board Slot Location